# Working on Agilent 8510C Network Analyzer

Initializing the (new) Network Analyzer :

- Press ‘Menu’ key in the Function block ‘STIMULUS’ of the Network Analyzer.

This presents soft key menu containing additional function labels on the CRT.

- Press the soft key to the right of the function label STEP in the soft key menu.

- Then press soft key to the right of the function label ‘NUMBER of POINTS’ in the soft key menu, which will display one more menu with various points. Press the soft key to the right of the number 51.

- Now press ‘SAVE’ key in the INSTRUMENT STATE Block that will bring Instrument State select menu onto the CRT display. Press the soft key to the right of any one of the numbers on the CRT display to save the current complete state of the network analyzer in the corresponding storage register.

(Presently I saved in number 8).

Electrical delay is 0.063 nano sec.