Types of Problems

- STATIC

- DC – Potential between two conductors, capacitance

- Steady-state sinusoidal – incident field or current is sinusoidal (sin(wt)), and solution of steady-state equation is sought (eg.  antenna radiation pattern, field or current distributions)

- DYNAMIC

- Transient – such as pulse traveling down a microstripline, synthetic aperture radar pulses, coupling of pulses to scatterers/absorbers

- Steady state – solutions over a broad frequency band may be obtained by taking Fourier Transform of wideband transient solution

Types of Solutions

- Analytical

- Advantage – “exact” solution

- Disadvantage – can only be done for uniform, simple geometries

- Numerical Methods

- Advantage – can be done for complex, heterogeneous geometries

- Disadvantage – approximation (hopefully a good one) to the exact solution

Procedure:

- Divide model into discrete “cells” / “points” / “grids”

- Apply Maxwell’s equations to this discretized model

- Solve by matrix methods of iteration

Methods Covered in this Course

- Finite-Difference Frequency Domain (FD or FDFD)

- Maxwell’s equations (differential form) for steady state are converted to a very sparse matrix equation and solved, usually using an iterative matrix equation solution method

- Static / Differential Form / Matrix Equation

- Method of Moments (Galerkin Method, Method of Weighted Residuals)

- Maxwell’s equations (integral form) for steady state are converted to matrix equation and solved.  This matrix equation is not generally sparse.

- Ideal for non-absorbing targets (metal objects), wires, layered materials, small heterogeneities imbedded in large, predictably simple structures.

- Static / Integral Form / Matrix Equation

- Finite Element Method (FEM)

- Energy in system is minimized, and resulting sparse matrix equations are solved for steady state solutions.

- Ideal for geometries where curved edges matter.

- Static (or iteratively solved, dynamic) / Integral Form / Matrix Equation

- Finite Difference Time Domain (FDTD)

- Maxwell’s Equations (differential form) in the time domain are solved iteratively for transient solution.

- Steady State values are obtained from Fourier Transform of transient solution.

- Ideal for realistic time-domain simulation, highly heterogeneous models, particularly absorbers.   Method becomes more complicated for curved metal objects, and geometrical optics-sized scatterers.

- Dynamic / Differential Form / Iterative Solution

Other Topics

- Review of EM

- Review of Linear Algebra

- Methods of Solving Matrix Equations

- Numerical Integration

- Numerical Differentiation

- Time-to-Frequency Domain Conversion

Learning Objectives

- To become familiar with each method through understanding of the theory and programming it yourself

- Understand the basis for the advantages and disadvantages of each method

- Study the similarity of each method