Maxwell’s Equations - Point Form (Differential Form) Time Domain

^^^ Used for Discrete (numerical) solution

 X E = -  B

t

 X H = D  + Jsource

t

  B = 0

  D = q v source

Constituitive Relationships

Assuming “Simple Media”

- Uniform in space (at least the small discrete space where you are working)

- Single Frequency or Frequency Independent Properties

- NOT time varying

D = E

B = H

Units

E = V/m = N/C	Electric Field

D = C/m2 		Electric Flux Density

H = A/m		Magnetic Field

B = Wb/m2 = Tesla = N/(A*m)  Magnetic Flux Density

q v = C/m3		Charge Density (volume)

J = A/m		Current Density

 = x /x +   y /y +   z /z

Maxwell’s Equations:  Static Form

Substitute /t = 0 in above equations.

X E = 0 	(1)

X H = J

 H = 0

 E = q v /	 (2)

Define Potential =  (volts)

E = -  

Substitute into (1) and (2) above.

 X (-) = 0 which satisfies the vector identity:  x A = 0

(- ) = q v/

Which gives:

Poisson’s Equation:

2 = - q v / 

Laplace’s Equation (Poisson’s for a source-free region)

2 = 0

Maxwell’s Equations (Dynamic Forms)

q v = 0 (no free charges floating around)

- TRANSIENT:

NO further assumptions.  Solve the above Maxwell’s Equations in the time domain.

- STEADY STATE:

Assume sinusoidal fields:  E(t) = Eo ejt

E(t) = jEo ejt = j E(t)

t

Assume Simple Media:  D = E, B = H

X E = -jH 		(1)

 X H = jE + Jsource 	(2)

  H = 0

  E = 0

Solve for H in (1):  H = -  1       X E

j

Plug into (2):  -  1   X   X E = jE + Jsource

j

Use vector identity:  X  X E =  ( E) -  2 E

 2 = 2  + 2  + 2  		^^=0

x2   y2      z2

Gives Vector Wave Equation

(2 - 2 ) E = j Jsource

wavenumber: k =   () = 2 / 

wavelength: = c / f = 2  c  / 

speed of propagation : c = co /  ( r )  =  1/ (oor)  = 1/ ()

This gives the Inhomogenous vector wave equation:

(2 - k2) E = j Jsource

For a source-free region, we have the Homogeneous vector wave equation

(2 - k2) E = 0

For the homogeneous case, the vector equation can be divided into independent x,y,z components, so becomes a scalar wave equation

(2 - k2) Ex = 0

(2 - k2) Ey = 0

(2 - k2) Ez = 0