Name

Name

UNIVERSITY OF UTAH

ELECTRICAL AND ENGINEERING DEPARTMENT

ECE 5325

WIRELESS TRANSMISSION SYSTEMS

MIDTERM #2

YOU MAY USE A CALCULATOR & PORTFOLIO

NOTE:  Erlang B and Q charts are available on the next 2 pages of this exam

April 18,2005

Potentially Helpful Tables and Graphs.  If you use these charts, please MARK the places where you obtain your data.

Figure F.2 Plot of the Q Function

1.(33 points)  Find the cell radius for transmission from the mobile to the base station given the following:

power of the mobile = 600 mW

power allocated to the pilot channel = 15 mW

The interference must be at least 7 dB below the energy in each bit.

The noise figure of the cell = 5 dB

bandwidth = 2 MHz

chip rate = 1 MHz

data rate = 100 kHz

The cell is in use 40 % of the time.

Two high-gain directional antennas are used to transmit over the same

path as the cell.  The transmitting antenna sends 0 dBm of power, and

-120 dBm is received by the other antenna 1 meter away.

Path loss exponent = 4, environment is typical outdoor environment (T =

290 degrees K)

Hint: Get started on the right foot:

Are you calculating the FORWARD or REVERSE link?

2.(33 points)  How much other cell interference can you tolerate for the IS-95 system  described below.  INCLUDE units in W and dB in your answer.

IS-95 bandwidth = 1.23 MHz

IS-95 data rate = 9.6 kbps

Power of the mobile = 100 mW

Miscellaneous losses =  8 dB

Two dipole antennas are used to measure that the power transmitted over 1

m is 0 dBm.  When the high gain base station and mobile antennas are used with the

same input power levels and same distance, 10 dBm is measured.

Path loss = 150 dB

Normal outdoor operating environment: depth of fade = 20 dB, T = 290 K

Noise figure of base station = 5 dB

System is used 40 of the time.

Number of mobile users is 25

3.(33 points) How many users can an IS95 cell support given the following  conditions:

Probability that a call will be blocked = 2  %

Fading Margin = 20 dB

Path Loss Exponent = 4

Standard Deviation of the path loss = 4

Cell uses 2-way soft handoff

Information about the  IS-95 Standard:

Data rate = 9.6 kbps

Minimum Eb/It  = 7 dB

Chip Rate = 1.23 MHz

The system is active 40 % of the time

1/η = 10

Standard Deviation of Power Control = 3 dB

ECE 5325

Midterm #3

Name

SCORE:

Problem 1  		  of a possible  33  points

Problem 2  		  of a possible  33   points

Problem 3  		  of a possible  33  points

+1

Total       		  of a possible 100 points