Name

Name

UNIVERSITY OF UTAH

ELECTRICAL AND ENGINEERING DEPARTMENT

ECE 5960

WIRELESS TRANSMISSION SYSTEMS

MIDTERM #1

YOU MAY USE A CALCULATOR & PORTFOLIO

February 7, 2003

1.	(33 points)

A hexagonal cell within a seven-cell cluster has a radius of 1 km.  The cluster is allocated a frequency band of 100 MHz, and each channel is 1 MHz wide.  The average user makes one 10-minute call per hour.

a.	What is the load per user?

b.	How many users can the cluster support while having a 1% probability of delay?

c.	What is the probability the call will be delayed more than 10 seconds?

2.	(33 points) A person is sitting inside a car talking on a cell phone.  Determine how many watts of power that should be transmitted by the base station given:

Bandwidth = 0.5 MHz

Temperature = 294K

Noise figure of amplifier = 2 dB

Required S/N = 10 dB

Power lost in body = 50%

Reflection coefficient at car roof = 1/3

Mobile antenna is a dipole.

Base station antenna has a gain of 6 dBd

Loss in base station cable = 2 dB

Mobile can transmit 0.6 W of power

Show all of your work.

Power = ___________________ Watts

3.	(33 points)

A 0.6 W cell phone transmitter is located 1 km from the base station.  The frequency is 915 MHz, and the bandwidth is 1 MHz.

a.	How much power does the base station receive according to the Friis transmission equation assuming no additional losses or gains?

b.	How much power does the base station receive according to the Okumura model in a suburban area where the mobile is 1.5 m high, and the base station is 10 m high?

ECE 5960

Midterm #1

February 7, 2003

Name 							Random Student Number ________

SCORE:

Problem 1  		  of a possible  33  points

Problem 2  		  of a possible  33   points

Problem 3  		  of a possible  33  points

+1

Total       		  of a possible 100 points