Shannon Capacity

Q1: Reformulate (7.83) to obtain Eb/No in terms of Rb/B at the Shannon limit.  Hint: Rb = C at the Shannon limit.

Q2: Plot Rb/B vs Eb/No (dB) from Eb/No = [ -2 … 4 ] dB.

Q3: Compute the following values for uncoded, 1 Mbps BPSK and add the data point to your plot in Q2:

- At what Eb/No is the BER = 10-6?

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/Nouncoded_BPSK, Rb/Buncoded_BPSK ) to your plot in Q2.

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q4: Compute the following values for uncoded, 1 Mbps QPSK and add the data point to your plot in Q2:

- At what Eb/No is the BER = 10-6?

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/Nouncoded_QPSK, Rb/Buncoded_QPSK ) to your plot in Q2.

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q5: Compute the following values for uncoded, 1 Mbps 8-PSK and add the data point to your plot in Q2:

- At what Eb/No is the BER = 10-6?

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/Nouncoded_8-PSK, Rb/Buncoded_8-PSK ) to your plot in Q2?

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q6: Compute the following values for (7, 4)-Hamming encoded, 1 Mbps QPSK and add the data point to your plot in Q2:

To achieve BER = 10-6, this code requires Eb/No = 10 dB.

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/No(7, 4)_QPSK, Rb/B(7, 4)_QPSK ) to your plot in Q2.

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q7: Compute the following values for (31, 26)-Hamming encoded, 1 Mbps QPSK and add the data point to your plot in Q2:

To achieve BER = 10-6, this code requires Eb/No ~ 9 dB.

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/No(31, 26)_QPSK, Rb/B(31, 26)_QPSK ) to your plot in Q2.

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q8: Compute the following values for (127, 64)-BCH, 1 Mbps QPSK and add the data point to your plot in Q2:

To achieve BER = 10-6, this code requires Eb/No ~ 6.5 dB.

- Assuming root raised cosine filtering with alpha = 0.2, what is B?

- Add this point ( Eb/NoBCH_QPSK, Rb/BBCH_QPSK ) to your plot in Q2.

- How many dB away from capacity is this modulation (for a fixed Rb/B)?

Q9: Which of these codes/modulations is closest to the Shannon capacity bound?

Now, open the following simulink demo:

Blocksets -> Communications -> Application-Specific Examples -> DVB-S.2 Link, Including LDPC Coding

Right-click the orange “Settings” block and select “Edit Mask.”

Q10: What function gets called during initialization?

At the matlab command prompt, type:	edit <answer to Q10>

You’ll find the following line:

LDPCParityCheckMatrix = dvbs2ldpc(str2num(codeRate));

Execute a similar line:

H = dvbs2ldpc( 1/2 );

Q11: Describe this code:

- How big is H?   (Use size(H))

- How many LDPC iterations will the decoder make?  (double-click the settings box in Simulink.)

- Is this code likely to outperform the Hamming or BCH codes considered above?  At what cost?

Q12: Run the simulation with the default settings.  Then fill in the following chart.  Assume the Since the BCH rate is always between 0.98 and 1 in DVB-S2, assume it’s 1.  Then select the various modulation/LDPC coding rate pairs listed in the table.

- Run the simulation at various Eb/No levels in steps of 0.5 dB to determine the lowest Es/No level for which the simulation runs error free.

- Compute the remaining values in the table, but save the last column until after (c).

- Add these Rb/B vs Eb/No values to your plot in Q2.

- Using (c), determine how far this codec is from Shannon capacity.

- How much better is this code than the code you chose in Q9?

- Why do you think DVB-S2 has so many rate choices?

| Mod | Code rate | Es/No req’d for BER = 0, 0.5 dB resolution | Eb/No | B (Mbps) given Rb = 1 Mbps, alpha = 0.2 | Rb/B | “distance” to Shannon capacity curve in dB |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1/4 |  |  |  |  |  |
| QPSK | 1/3 |  |  |  |  |  |
| QPSK | 1/2 | 1 | 1 | 1.2 | 0.83 | ~1 |
| QPSK | 3/4 |  |  |  |  |  |
| QPSK | 9/10 |  |  |  |  |  |
| 8-PSK | 3/5 |  |  | 0.67 |  |  |
| 8-PSK | 9/10 |  |  |  |  |  |

Q13: Which data point achieved the highest spectral efficiency (Rb/B)?  How much more energy (Eb/No) does this choice require than un-coded QPSK?

Q14: Which data point achieved the best power efficiency (lowest Eb/No)?  How much more bandwidth does it require than un-coded QPSK?