% generate and analyze Rayleigh and Rician data

% generate Rayleigh data

N = 10000;

sig = 1.5;

rayls = raylrnd( sig, N, 1 );

% find pdf

[ pdf_unscaled, r_rayl ] = hist ( rayls, 50 );

pdf_rayl = pdf_unscaled / N / (r_rayl(2)-r_rayl(1));

figure(1); plot( r_rayl, pdf_rayl );

rayl_mean = mean( rayls );

sig_est = rayl_mean / 1.2533;

curve_est = r_rayl/sig_est^2 .* exp( - r_rayl.^2 / (2*sig_est^2) );

figure(2); plot( r_rayl, pdf_rayl, 'b-x', r_rayl, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

% however things scale differently if there's a power offset:

data = 20*log10( rayls ) - 20;

figure(3); plot( data )

vs = 10 .^ ( data/20 );

vs_norm = vs / mean( vs );

[ pdf_unscaled_data, r_data ] = hist ( vs_norm, 50 );

pdf_data = pdf_unscaled_data / N / (r_data(2)-r_data(1));

figure(4); plot( r_data, pdf_data );

rayl_mean = mean( vs_norm );

sig_est = rayl_mean / 1.2533;

curve_est = r_data/sig_est^2 .* exp( - r_data.^2 / (2*sig_est^2) );

figure(5); plot( r_data, pdf_data, 'b-x', r_data, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

% suppose the values aren't Rayleigh...

ricis = simout.signals.values(1:end-1);

[ pdf_unscaled_ricis, r_ricis ] = hist ( ricis, 50 );

pdf_ricis = pdf_unscaled_ricis / N / (r_ricis(2)-r_ricis(1));

figure(6); plot( r_ricis, pdf_ricis );

rayl_mean = mean( ricis );

sig_est = rayl_mean / 1.2533;

curve_est = r_ricis/sig_est^2 .* exp( - r_ricis.^2 / (2*sig_est^2) );

figure(7); plot( r_ricis, pdf_ricis, 'b-x', r_ricis, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

ricis2 = simout1.signals.values(1:end-1);

[ pdf_unscaled_ricis2, r_ricis2 ] = hist ( ricis2, 50 );

pdf_ricis2 = pdf_unscaled_ricis2 / N / (r_ricis2(2)-r_ricis2(1));

figure(16); plot( r_ricis2, pdf_ricis2 );

rayl_mean = mean( ricis2 );

sig_est = rayl_mean / 1.2533;

curve_est = r_ricis2/sig_est^2 .* exp( - r_ricis2.^2 / (2*sig_est^2) );

figure(17); plot( r_ricis2, pdf_ricis2, 'b-x', r_ricis2, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

ricis3 = simout2.signals.values(1:end-1);

[ pdf_unscaled_ricis3, r_ricis3 ] = hist ( ricis3, 50 );

pdf_ricis3 = pdf_unscaled_ricis3 / N / (r_ricis3(2)-r_ricis3(1));

figure(26); plot( r_ricis3, pdf_ricis3 );

rayl_mean = mean( ricis3 );

sig_est = rayl_mean / 1.2533;

curve_est = r_ricis3/sig_est^2 .* exp( - r_ricis3.^2 / (2*sig_est^2) );

figure(27); plot( r_ricis3, pdf_ricis3, 'b-x', r_ricis3, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

ricis4 = simout2.signals.values(1:end-1);

[ pdf_unscaled_ricis4, r_ricis4 ] = hist ( ricis4, 50 );

pdf_ricis4 = pdf_unscaled_ricis4 / N / (r_ricis4(2)-r_ricis4(1));

figure(26); plot( r_ricis4, pdf_ricis4 );

rayl_mean = mean( ricis4 );

sig_est = rayl_mean / 1.2533;

curve_est = r_ricis4/sig_est^2 .* exp( - r_ricis4.^2 / (2*sig_est^2) );

figure(27); plot( r_ricis4, pdf_ricis4, 'b-x', r_ricis4, curve_est, 'r-' );

title( 'Rayleigh fit to data' );

legend( 'measured', 'curve fit' );

xlabel( 'r' );

ylabel( 'pdf( r )' );

% consider using the Rician, Rayleigh cdfs to fit to data cdf

s = sort( rayls );

median_rayl = s(N/2);

cdf_rayl = cumsum( pdf_rayl ) / sum(pdf_rayl);

s = sort( vs_norm );

median_data = s(N/2);

cdf_rayl_data = cumsum( pdf_data ) / sum(pdf_data);

s = sort( ricis );

median_ricis = s(N/2);

cdf_ricis = cumsum( pdf_ricis ) / sum(pdf_ricis);

s = sort( ricis2 );

median_ricis2 = s(N/2);

cdf_ricis2 = cumsum( pdf_ricis2 ) / sum(pdf_ricis2);

median_ricis3 = s(N/2);

cdf_ricis3 = cumsum( pdf_ricis3 ) / sum(pdf_ricis3);

median_ricis4 = s(N/2);

cdf_ricis4 = cumsum( pdf_ricis4 ) / sum(pdf_ricis4);

figure(8); semilogy( 20*log10(r_rayl/median_rayl), cdf_rayl, 'b-', ...

20*log10(r_data/median_data), cdf_rayl_data, 'rd', ...

20*log10(r_ricis/median_ricis), cdf_ricis, 'k-x', ...

20*log10(r_ricis2/median_ricis2), cdf_ricis2, 'k-s', ...

20*log10(r_ricis3/median_ricis3), cdf_ricis3, 'k--', ...

20*log10(r_ricis4/median_ricis4), cdf_ricis4, 'ko' );

axis( [ -20 10 1e-3 1 ] );

legend('Rayleigh, \sigma = 1.5', 'Rayleigh, \sigma = 1.5, -20 dB & normed', ...

'Rician, \sigma = 1.5, K = 2', 'Rician, \sigma = 1.5, K = 4', ...

'Rician, \sigma = 1.5, K = 10', 'Rician, \sigma = 4, K = 10' );

xlabel('Signal level (dB about median)');

ylabel('probability signal < abscissa');