Midterm 2 study guide:

Review sections:

5.1, 5.3-6,

6. 11,

7.1-2, 7.10-12

8.3, 8.7

MacKay’s treatment of I(x), H(x) 4.1-3

Here’s a start at some test-style sub-problems.  Obviously, not all of them will fit on a single test.  I’ll answer many of these during our review on Tuesday.  I’ll also hand calculate many of them and post the solutions online.

- Given m = 5, taps = [ 5 1 ], initialized with all zeros except for a single one at the far right, compute the first 6 values of this maximal length sequence.

- How long is the PN code in (1)?

- What does it look like if you correlate the sequence from (1) against a continuously repeating copy of itself?  How high and how far apart are any unusual parts of that correlation?

- If the processing gain in a spread spectrum system is 100, and there are eleven users in the system, what is the SIR?

- Draw a picture of the frequency-domain representation of the signal in (4) before and after spreading and despreading.

- Draw a picture of the impact of despreading on a narrow-band interferer.

- Use the following repeating sequence [ 1 1 -1 1 -1 -1 1 ] to spread the following bit stream [ 1 0 ].

- Draw a time-domain picture of the spread sequence from (6).

- Encode [ 0 1 0 0 1 1 1 0 ] using the (7,4) Hamming code.

- Decode the following symbol stream [ -.3 -.9 .8 -1.1 -.9 -1.0 -1.2 ] using the Hamming (7,4) code.  (Note the syndrome look-up table is [ 3 2 5 1 7 4 6 ].)

- What’s the likeliest bit to detect incorrectly in (9)?

- Compute a pdf and cdf from measured data as shown on Midterm #2 from 2005.

- Determine if a channel is frequency selective given sigma_T or Bc and Ts or Rs or Rb.

- Determine if a channel exhibits fast fading given Ts or Bs or Rs and the velocity and wavelength of the user or the maximum Doppler.

- Recommend a mitigation to frequency selective fading.

- Recommend a mitigation to slow fading.

- Given five branches of receive diversity, compute the probability that SNR_instantaneous / SNR_average < -8 dB under selection combining.

- Given instantaneous SNRs on five diversity branches of 10, 12, 8, 3, and 7 dB, compute the output SNR under selection combining.

- Compute the output SNR for (18) under maximal-ratio combining.

- Indicate the coding gain given curves for the BER, SER and decoded BER.

- Indicate the relative impact of code rate on BER

- Indicate the relative impact of code block size on BER.

- Compute R_FEC given n = 1000, k = 300.

- Compute Es given Eb, R_FEC = 0.4

- Compute Rs given R_FEC = 0.25, Rb.

- Given received values y = [ 1, 2, 3, 25 ] and an interleaver of size 5 x 5, show the output sequence.

- What assumptions make an interleaver of value in a system?

- Assume |H(f)|2 looks like two periods of a sinusoid.  Draw the frequency response of a corrective equalizer.

- Draw the combined response of the channel and equalization filter.

- Given ten transmit and receive values, compute Y, X and the equalization filter of length 3 from 5 of the X values.

- Compute how many bits are required to achieve an SQNR of 60 dB.

- State which of the following consonants will use a noise source excitation in an LPC vocoder: t, g, h, k, v, f.

- How much can a bit stream be compressed if p(x=1) = .3, p(x=0) = .7?

- What bandwidth will a spread spectrum system occupy if Bs = 100 kHz and PG = 50?

- Why would diversity be avoided on a particular system?

- Given an MRC or selection diversity combining chart, find the power received at least 97% of the time with 4-branch diversity.

- Compute the delay spread of four delta functions with magnitudes [ 1 .8 .7 .4 ] and delays [ 0 50 90 110 ] microseconds of relative delay.

- State whether a signal with Rb will see the channel in (37) as frequency selective or frequency flat.