Practice Problems

- A 8 V d.c. voltage source is applied across two parallel plates that are connected in parallel. Each plate has a surface area of 64 X10-2 cm2 . The two plates are separated by 4 cm.

    - Neglecting fringing and assuming free space between the conductors. Find the following:

- The electric field intensity E

ii) The capacitance between the conductors

iii) The energy stored.

B) If we replace the above coaxial cable with a conductor which is filled with a

dielectric with εr=2.56 and μr=1 in the half of the region and with free space in the

remaining area. Find the following:

- The electric field intensity E

- The capacitance between the conductors

- The energy stored.

- A 8 V d.c. voltage source is applied across the inner and outer conductors of a coaxial pipe with outer radius b=6 cm and inner radius a= 2cm as shown in the figure below. The length of the conductor l=6 cms.

    - Neglecting fringing and assuming free space between the conductors. Find the following:

- The electric field intensity E

ii) The capacitance between the conductors

iii) The energy stored.

B) If we replace the above coaxial cable with a conductor which is filled with a

dielectric with εr=2.56 and μr=1 in the region (π≤φ≤2π) and with free space in the

region (0≤φ≤π). Find the following:

- The electric field intensity E

- The capacitance between the conductors

- The energy stored.

3)  A plane wave has an electric field given by:

E=(30 {x−40 {z)e−j(40x+30z)

Find the following:

- The polarization of the wave

- Find wavelength and frequency for wave traveling in the media [ε,μ]= [4εo ,μo]

- The corresponding value of H

- The power density

- The phase constant β

4)  A plane wave has an electric field given by:

E=(10 {x−20 {y)e−j30z

Find the following:

- The polarization of the wave

- Find wavelength and frequency for wave traveling in the media [ε,μ]= [4εo ,μo]

- The corresponding value of H

- The power density

- The phase constant β

5) 6.25

6) 6.27